Department of Electrical Engineering

| Faculty Member:_________________ | Dated: ____________________ |
| --- | --- |
| Course/Section:BEE8 | Semester: Spring 2019 |

EE-330 Digital Signal Processing

Lab 5: Audio Processing using TMS 320C6713 DSK

|  |  | PLO4 |  | PLO5 | PLO8 | PLO9 |
| --- | --- | --- | --- | --- | --- | --- |
| Name | Reg. No | Viva / Quiz / Lab Performance | Analysis of data in Lab Report | Modern Tool Usage | Ethics and Safety | Individual and Team Work |
|  |  | 5 Marks | 5 Marks | 5 Marks | 5 Marks | 5 Marks |

# Lab5: Audio Processing using  TMS 320C6713 DSK

Objectives

The objective of this lab is to explore some more features of Code Composer Studio (CCS) that is time domain and frequency domain plots of audio signal   in addition to that we will also do real time processing of audio input.

- Time domain and frequency Domain Plots in CCS

- Real Time processing of  Audio Signal

- Working with basic sinusoids on DSP Kit

Lab Instructions

Lab Instructions

- The students should perform and demonstrate each lab task separately for step-wise evaluation (please ensure that course instructor/lab engineer has signed each step after ascertaining its functional verification)

- Each group shall submit one lab report on LMS within 6 days after lab is conducted. Lab report submitted via email will not be graded.

- . Students are however encouraged to practice on their own in spare time for enhancing their skills.

Lab Report Instructions

All questions should be answered precisely to get maximum credit. Lab report must ensure following items:

- Lab objectives

- C codes

- Results (graphs/tables) duly commented and discussed

- Conclusion

## Introduction

In this Lab we will explore some more features of Code Composer Studio that is  time domain and frequency domain plots of audio signal   in addition to that we will also  learn real time  processing   of audio input. We will also see the details of   support files that we used in previous Lab.

## SUPPORT FILES

The support files c6713dskinit.c ,vectors_intr.asm or vectors_poll.asm , and c6713dsk.cmd are used by nearly all of the examples .

### Initialization/Communication File ( c6713dskinit.c)

Source file c6713dskinit.c contains the definitions of a number of functions used to initialize the DSK. Calls are made from these functions to lower level functions provided with CCS in the board support library (BSL) and chip support library (CSL) files dsk6713bsl.lib and csl6713.lib . Functions comm_intr() and comm_poll() initialize communications between the C6713 processor and the AIC23 codec for either interrupt - driven or polling - based input and output. In the case of interrupt - driven input and output, interrupt  #11 (INT11), generated by the codec via the serial port (McBSP), is configured and enabled (selected). The nonmaskable interrupt bit must be enabled as well as the global interrupt enable (GIE) bit.Functionsinput_sample() , input_left_sample() , input_right_sample() , output_sample() , output_left_sample() , and input_right_sample() are used to read and write data to and from the codec. In the case of polling – based input and output, these functions wait until the next sampling instant (determinedby the codec) before reading or writing, using the lower level functions MCBSP_read() or MCBSP_write() . They do this by polling (testing) the receive ready (RRDY) or transmit ready (XRDY) bits of the McBSP control register (SPCR). In the case of interrupt - driven input and output, the processor is interrupted by the codec at each sampling instant and when either input_sample() or output_sample() is called from within the interrupt service routine, reading or writing proceeds without RRDY or XRDY being tested. Header File ( c6713dskinit.h)The corresponding header support file c6713dskinit.h contains function prototypes as well as initial settings for the control registers of the AIC23 codec. Nearly all of the example programs in  use the same AIC23 control register settings. However, two codec parameters — namely, sampling frequency and selectionof ADC input (LINE IN or MIC IN) — are changed more often, from one program example to another, and for that reason the following mechanism has been adopted. During initialization of the DSK (in function dsk_init() , defined in file c6713dskinit.c ), the AIC23 codec control registers are initialized using the DSK6713_AIC23_Config type data structure config defined in header file c6713dskinit.h . Immediately following this initialization, two functions DSK6713_AIC23_setFreq() and DSK6713_AIC23_rset() are called and these set the sampling frequency and select the input source according to the values of the variablesfs and inputsource . These values are set in the first few lines of every top level source file; for example,

Uint32 fs = DSK6713_AIC23_FREQ_8KHZ; //set sampling rate

#define DSK6713_AIC23_INPUT_MIC 0x0015

#define DSK6713_AIC23_INPUT_LINE 0x0011

Uint16 inputsource=DSK6713_AIC23_INPUT_MIC; //select input source

In this way, the sampling frequency and input source can be changed without having to edit either c6713sdkinit.h or c6713dskinit.c .

Function  defined  in these files

void c6713_dsk_init();

voidcomm_poll();

voidcomm_intr();

voidoutput_sample(int);

voidoutput_left_sample(short);

voidoutput_right_sample(short);

Uint32 input_sample();

shortinput_left_sample();

shortinput_right_sample();

### Vector Files ( vectors_intr.asm, vectors_poll.asm )

To make use of interrupt INT11, a branch instruction (jump) to the interrupt service routine (ISR) c_int11() defined in a C program, for example, sine8_buf.c , must  be placed at the appropriate point in the interrupt service table (IST). Assembly language file vectors_intr.asm , which sets up the IST, Note the underscore preceding the name of the routine or function being called. By convention, this indicates a C function. For a polling - based program, file vectors_poll.asm is used, in place of vectors_intr.asm . The main difference between these files is that there is no branch to c_int11() in the IST set up by vectors_poll.asm . Common to both files is a branch to c_int00() , the start of a C program, associated with reset.

### Linker Command File ( c6713dsk.cmd)

Linker command file C6713dsk.cmd specifies the memory configuration of the internal and external memory available on the DSK and the mapping of sections of code and data to absolute addresses in that memory. For example, the .text section, produced by the C compiler, is mapped into IRAM, that is, the internal memory of the C6713 digital signal processor, starting at address 0x00000220 . The section .vectors created by vectors_intr.asm or by vectors_poll.asm is mapped into IVECS, that is, internal memory starting at address

Generation of Sinusoid and Plotting with CCS ( sine8_buf )

This example generates a sinusoidal analog output signal using eight precalculated and prestored sample values. However, it differs fundamentally from sine8_LED in that its operation is based on the use of interrupts. In addition, it uses a buffer to store the BUFFERLENGTH most recent output samples. It is used to illustrate the capabilities of CCS for plotting data in both time and frequency domains. All the files necessary to build and run an executable file sine8_BUF.out are stored in folder sine8_buf . Program filesine8_buf.c is listed in Figure 3.12 . Because a project file sine8_buf.pjt is supplied, there is no need to create a new

//sine8_buf.c sine generation with output stored in buffer

#include "DSK6713_AIC23.h" //codec support

Uint32 fs=DSK6713_AIC23_FREQ_8KHZ; //set sampling rate

#define DSK6713_AIC23_INPUT_MIC 0x0015

#define DSK6713_AIC23_INPUT_LINE 0x0011

Uint16 inputsource=DSK6713_AIC23_INPUT_MIC; // select input

#define LOOPLENGTH 8

#define BUFFERLENGTH 256

intloopindex = 0; //table index

intbufindex = 0; //buffer index

shortsine_table[LOOPLENGTH]={0,707,1000,707,0,-707,-1000,-707};

intout_buffer[BUFFERLENGTH]; //output buffer

short gain = 10;

interrupt void c_int11() //interrupt service routine

shortout_sample;

out_sample = sine_table[loopindex++]*gain;

output_left_sample(out_sample); //output sample value

out_buffer[bufindex++] = out_sample; //store in buffer

if (loopindex>= LOOPLENGTH) loopindex = 0; //check end table

if (bufindex>= BUFFERLENGTH) bufindex = 0; //check end buffer

return;

} //return from interrupt

void main()

{

comm_intr(); //initialise DSK

while(1); //infinite loop

}

FIGURE 3.12. Listing of program sine8_buf.c .project file, add files to it, or alter compiler and linker build options. In order tobuild, download and run program sine8_buf.c .

### How to import existing project  CCS Project:

- Close any open projects in CCS.

- Go to  file   import and  you will see following figure

- Select CCS projects and click next

- Click browse and you will see following figure. Select   the root folder of the which is sine8_buff in our case.

5.Click finish and  you should be able to see the and relevant files of the prohect in project view.

Sine8_buff project description

Because this program uses interrupt - driven input/output rather than polling, the file vectors_intr.asm is used in place of vectors_poll.asm . The interrupt service table specified in vectors_intr.asm associates the interrupt service routine c_int11() with hardware interrupt INT11, which is asserted by the AIC23 codec on the DSK at each sampling instant. Within function main() , function comm_intr() is used in place of comm_poll().Essentially, it initializes the DSK hardware, including the AIC23 codec, such that the codec sampling rate is set according to the value of the variable fs and the codec interrupts the processor at every sampling instant. The statement while(1) in function main() creates an infinite loop, during which the processor waits for interrupts. On interrupt, execution proceeds to the interrupt service routine (ISR) c_int11() , which reads a new sample value from the array sine_table and writes it both to the array out_buffer and to the DAC using function output_left_sample() . Interrupts are discussed in more detail in Chapter 3 .Build this project as sine8_buf . Load and run the executable filesine8_buf. outand verify that a 1 - kHz sinusoid is generated at the LINE OUT and HEADPHONE sockets

### Graphical Displays in CCS

The array out_buffer is used to store the BUFFERLENGTH most recently output sample values. Once program execution has been halted, the data stored in out_ buffer can be displayed graphically in CCS.

1. Select ToolGraph Time signal and set the Graph Property Dialog properties as shown in Figure 3.13 a. Figure 3.13 b shows the resultant Graphical Display window.

2. Figure 3.14 a shows the Graph Property Dialog window that corresponds to the frequency domain representation of the contents of out_buffershown in Figure 3.14 b. The spike at 1 kHz represents the frequency of the sinusoid generated by program sine8_buf.c.

FIGURE 3.13. (a) Graph Property window and (b) Time domain plot of data stored inout_buffer.

FIGURE 3.14. (a) Graph Property window and (b) Frequency domain plot of data storedin out_buffer.

### Viewing and Saving Data from Memory into File

To view the contents of out_buffer , select View MemoryBrowser. Specify out_buffer as the Address and select 32 - bit Signed Integer as the Format , as shown in Figure 3.15 a.

FIGURE 3.15. (a)  Memory window view of data stored inout_buffer.

//sine8_buf.c sine generation with output stored in buffer

#include "DSK6713_AIC23.h"	                //codec support

Uint32 fs=DSK6713_AIC23_FREQ_24KHZ;	        //set sampling rate

#define DSK6713_AIC23_INPUT_MIC 0x0015

#define DSK6713_AIC23_INPUT_LINE 0x0011

Uint16 inputsource=DSK6713_AIC23_INPUT_MIC; // select input

#define LOOPLENGTH 8

#define BUFFERLENGTH 256

int loopindex = 0;   						//table index

int bufindex = 0;                           //buffer index

short sine_table[LOOPLENGTH]={0,707,1000,707,0,-707,-1000,-707};

int out_buffer[BUFFERLENGTH];               //output buffer

short gain = 10;

interrupt void c_int11()                    //interrupt service routine

{

short out_sample;

out_sample = sine_table[loopindex++]*gain;

output_left_sample(out_sample);	       //output sample value

out_buffer[bufindex++] = out_sample;     //store in buffer

if (loopindex >= LOOPLENGTH) loopindex = 0; //check for end of table

if (bufindex >= BUFFERLENGTH) bufindex = 0; //check for end of buffer

return;					               //return from interrupt

}

void main()

{

comm_intr();	//initialise DSK

while(1);     //infinite loop

}

## LAB TASK 1

Modify program sine8_buf.c to generate a sine wave with a frequency of 3000 Hz. Verify your result using an oscilloscope connected to the LINE OUT socket on the DSK as well as using Code Composer to plot the 32 most recently output samples in both the time and frequency domains.

Uint32 fs=DSK6713_AIC23_FREQ_24KHZ;	        //set sampling rate

## LAB TASK 2

Write polling - based program such that when DIP switch #3 is pressed down, LED #3 turns on and a 500 - Hz cosine wave is generated for 5 seconds.

//sine8_LED.c  sine generation with DIP switch control

#include "dsk6713_aic23.h"  	       //codec support

Uint32 fs = DSK6713_AIC23_FREQ_8KHZ;   //set sampling rate

#define DSK6713_AIC23_INPUT_MIC 0x0015

#define DSK6713_AIC23_INPUT_LINE 0x0011

Uint16 inputsource=DSK6713_AIC23_INPUT_MIC;//select input

#define LOOPLENGTH 16

short loopindex = 0;         	      //table index

short gain = 100;			          //gain factor

short sine_table[LOOPLENGTH]={1, 0.92, 0.707, 0.38, 0, -0.38, -0.707, -0.92, -1, -0.92, -0.707, -0.38, 0, 0.38, 0.707, 0.92}; //sine values

void main()

{

comm_poll();              	      //init DSK,codec,McBSP

DSK6713_LED_init();			      //init LED from BSL

DSK6713_DIP_init();			      //init DIP from BSL

/*while(1) 				              //infinite loop

{

if(DSK6713_DIP_get(0)==0) 		  //=0 if DIP switch #0 pressed

{

DSK6713_LED_on();	   		      //turn LED #0 ON

output_left_sample(sine_table[loopindex++]*gain); //output sample

if (loopindex >= LOOPLENGTH) loopindex = 0; //reset table index

}

else DSK6713_LED_off(0); 		  //turn LED off if not pressed

}          */                         //end of while(1) infinite loop

int i;

while (1)

{

if(DSK6713_DIP_get(0)==0)

{

for (i = 0; i < 40000; i++)

{

DSK6713_LED_on();	   		      //turn LED #0 ON

output_left_sample(sine_table[loopindex++]*gain); //output sample

if (loopindex >= LOOPLENGTH) loopindex = 0; //reset table index

}

}

else

DSK6713_LED_off(0); 		  //turn LED off if not pressed

}

}					                  //end of main

## LAB TASK 3

Write an interrupt - driven program that maintains a buffer containing the 128 most recent input samples read at a sampling frequency of 16 kHz from the AIC23 codec, using the MIC IN socket on the DSK. Halt the program andplot the buffer contents using Code Composer.